Step 2a  -  H10 - Higher capacity mineral drying

Market scan

Market size

Same underlying pool as H9: mineral drying equipment+services is order-of-magnitude ~$1-3B/yr globally inside a ~$5-10B industrial dryer market, with minerals/ores ~24% of rotary dryer demand. The capacity-booster angle targets the installed base rather than new lines: dryers run 20-40 years, replacement/upgrade is a named demand driver in market reports, and OEM aftermarket/service is ~30-40% of dryer-OEM revenue (~). A new large rotary line costs ~$8-15M installed with 12-18-month lead times, so a retrofit that adds ~20-30% throughput for low single-digit $M addresses a real capex-avoidance budget — adjacent precedents exist (microwave boost ovens +25% throughput in food; IR booster sections debottlenecking paper machines; parallel fluid-bed additions at a 48->100 ktpa magnesium plant).

| Region | Size ($B) | Region | Size ($B) |
| --- | --- | --- | --- |
| Europe | ~$0.3-0.7B (aging installed base + ETS pressure; retrofit-friendly, capex-constrained mid-size producers) | Asia | ~$1-2B (largest market but dominated by new-build/greenfield; retrofit slice smaller share) |
| America | ~$0.5-1.2B (large aging fleet; debottlenecking culture strong in NA minerals/mining) | Africa | ~$0.1-0.3B (mine expansions; often cheaper to overbuild new than retrofit) |

Target customers

Operators whose drying step limits line throughput — typically because feed moisture rose (wetter ore, seasonal stockpiles), demand grew, or the dryer aged. Same base as H9: ~750 IMA-member processing plants in Europe, majors like Omya (~160 plants) and Imerys (~100+ sites) centralizing engineering decisions, plus hundreds of ore concentrators running single-line dryers where +10-20% capacity has high marginal value against $8-15M for a parallel line. Reachable through dryer-OEM aftermarket channels, plant engineering managers, and industry consultants; buying trigger is a capacity project already being scoped, so timing windows are episodic but the value case (avoided capex + deferred permitting) is concrete.

| Customer size | Approx. number | Market share (in %) |
| --- | --- | --- |
| Large | ~30-50 producer groups worldwide, ~500-800 drying plants (~) | ~50-60% of dried mineral tonnage (~) |
| Medium | ~300-500 regional producers, ~1,000-2,000 plants (~) | ~30% of tonnage (~) |
| Small | ~1,000-3,000 single-site quarry/processors (~) | ~10-15% of tonnage (~) |

Players offering a solution today

Conventional debottlenecking is served by the incumbent dryer OEMs' aftermarket arms (Metso, FLSmidth, Andritz, ThyssenKrupp, GEA) selling upgrades, heat-recovery retrofits (~18-25% fuel savings reported in EU cement/mineral retrofits) and parallel units, plus mid-tier OEMs doing add-on dryers. Booster/inline technology is a niche: microwave boost ovens proven in food (Ferrite/IMS MIP-9, +25% throughput), IR booster sections proven in paper/board (Ircon Solaronics), but no established microwave-booster player for bulk minerals — whitespace similar to H9.

| Player | Type of solution | Market share (in %) |
| --- | --- | --- |
| Large OEM aftermarket (Metso, FLSmidth, Andritz, ThyssenKrupp, GEA) | dryer upgrades, indirect-heat & heat-recovery retrofits, parallel lines, service contracts | ~35-50% of new dryer revenue; aftermarket ~30-40% of OEM sales (~) |
| Medium (Allgaier, Comessa, Schenck, Carrier Vibrating, FEECO, Schwing Bioset, Hazemag, Louisville) | add-on rotary/FBD dryers, refurbishment, regional debottlenecking projects | ~30-40% (~) |
| Small booster/dielectric niche (Ferrite/IMS, SAIREM, Pueschner, Cellencor; IR specialists e.g. Ircon Solaronics) | microwave/IR booster and pre-dryer modules 6-300 kW; proven +25-30% throughput in food/paper, mineral refs scarce | <5-10% today, emerging (~) |

Regulation

| Regulation | Jurisdiction | What it means for us |
| --- | --- | --- |
| EU ETS (Directive 2003/87/EC) | EU | Debottlenecking by adding gas-fired capacity increases ETS exposure (~EUR60-90/tCO2); an electric booster adds throughput with zero on-site combustion. |
| MCPD 2015/2193 / IED permit review | EU | Uprating burners or adding combustion capacity can trigger permit modification and tighter emission limits; an electric booster avoids reopening combustion permitting. |
| US EPA NSPS/NESHAP air permitting | US | Increased firing rates on existing dryers can trigger New Source Review/BACT analysis; non-combustion boosters sidestep it. |
| Energy Efficiency Directive (EU) 2023/1791 | EU | Mandatory energy audits push operators to exploit low-capex efficiency/capacity measures — a booster that raises output per GJ is a natural audit action. |
| National decarbonization funding (DE EEW, EU Innovation Fund, US IRA 48C/DOE) | EU, DE, US | Retrofit projects combining capacity gain with electrification qualify for process-heat decarbonization subsidies, improving booster economics. |

| Personal input |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | Devin (agent draft) |  | Date | 2026-10-30 |  | Name |  |  | Date |  |
| Decision |  | ☒ Go to next step |  |  | ☐ Rework |  |  | ☐ Drop the hypothesis |  |  |